Tender Specifications

The “AEL-AE5” application includes the following elements:

• N-ALI01. Industrial Main Power Supply Module.

Supply voltage: 400 VAC, 3PH + N.

ON / OFF removable key.

Output voltage connections:

Three-phase + Neutral: 400 VAC.

Single-phase: 230 VAC.

Three-phase supply hose with IP44 3PN+E 32 A 400 V connecting plug.

Differential magnetothermal 4 poles, 25 A, 30 mA AC 6 KA.

Emergency stop push-button.

• N-ALI03. AC Auxiliary Power Supply (24 Vac) Module.

Supply voltage (Single-Phase):  230 VAC, 1PH + N.

Output voltage:

Single-Phase 24 VAC / 12 VAC.

24 VDC.

0 – 24VDC through potentiometer.

• N-CON01. Three-pole contactor module (24 VAC).

Nominal voltage for power contacts: 400 VAC.

Nominal voltage for control contacts: 24 VAC.

Nominal voltage for the control coil: 24 VAC.

Contacts:

Three-phase Normally Open Contact (NO) for power circuit.

Three Normally Open Contacts (NO) for control circuit.

Two Normally Close Contacts (NC) for control circuit.

Ground terminal.

• N-REF. Single-Phase Fixed Resistance Module (4 units).

Value: 150 ohm.

Maximum power: 500 W.

Switch:

Position 0: circuit opened.

Position 1: circuit closed.

Fuse: 2 A.

GND terminal.

• N-REL23/A. Ground Current Shunt Relay Module.

Power switch.

Earth leakage relay.

Reset push-button.

Test push-button.

Contacts:

Two Normally Closed contact (NC).

Indicator lamp.

Terminals:

Toroid connection.

Power supply.

Ground.

• N-REL23/B. Overcurrent Relay Module.

Power switch.

Latch switch.

Over current relay.

Normally Closed contact (NC).

Terminals:

Three input terminals.

Output terminal.

Power supply.

Ground.

• N-TRA10. 25/5 A Current Transformer Module.

Current transformer.

Ratio: 25/5.

Ground terminal.

• N-TRA31. 1/1000 A Current Transformer Module.

Current transformer.

Transformation ratio: 1000/1.

GND terminal.

• All necessary cables to realize the practical exercises are included.

Cables and accessories, for normal operation.

Manuals:

This unit is supplied whit the following manuals: Required services, Assembly and Installation, Starting-up, Safety, Maintenance & Practices manuals.

-Dimensions: 640 x 320 x 670 mm approx. (25.19 x 12.59 x 26.37 inches approx.) - Weight: 60 Kg approx. (132 pounds approx.)

EXERCISES AND PRACTICAL POSSIBILITIES

1.-  Checking the woking of the main power supply.

2.-  Checking the woking of the auxiliary power supply.

3.-  Start up of a three-phase contactor.

4.-  Calculation of the transformation ratio of a toroid.

5.-  Start up of an overcurrent relay.

6.-  Start up of an earth leakage relay.

Optional

AEL-AE5/ICAI. Interactive Computer Aided Instruction Software:

This complete software package consists of an Instructor Software (EDIBON Classroom Manager - ECM-SOF) totally integrated with the Student Software (EDIBON Student Labsoft - ESL-SOF). Both are interconnected so that the teacher knows at any moment what is the theoretical and practical knowledge of the students.

- ECM-SOF. EDIBON Classroom Manager (Instructor Software).

ECM-SOF is the application that allows the Instructor to register students, manage and assign tasks for workgroups, create own content to carry out Practical Exercises, choose one of the evaluation methods to check the Student knowledge and monitor the progression related to the planned tasks for individual students, workgroups, units, etc... so the teacher can know in real time the level of understanding of any student in the classroom.

Innovative features:

• User Data Base Management.

• Administration and assignment of Workgroup, Task and Training sessions.

• Creation and Integration of Practical Exercises and Multimedia Resources.

• Custom Design of Evaluation Methods.

• Creation and assignment of Formulas & Equations.

• Equation System Solver Engine.

• Updatable Contents.

• Report generation, User Progression Monitoring and Statistics.

- ESL-SOF. EDIBON Student Labsoft (Student Software).

ESL-SOF is the application addressed to the Students that helps them to understand theoretical concepts by means of practical exercises and to prove their knowledge and progression by performing tests and calculations in addition to Multimedia Resources. Default planned tasks and an Open workgroup are provided by EDIBON to allow the students start working from the first session. Reports and statistics are available to know their progression at any time, as well as explanations for every exercise to reinforce the theoretically acquired  technical knowledge.

Innovative features:

• Student Log-In & Self-Registration.

• Existing Tasks checking & Monitoring.

• Default contents & scheduled tasks available to be used from the first session.

• Practical Exercises accomplishment by following the Manual provided by EDIBON.

• Evaluation Methods to prove your knowledge and progression.

• Test self-correction.

• Calculations computing and plotting.

• Equation System Solver Engine.

• User Monitoring Learning & Printable Reports.

• Multimedia-Supported auxiliary resources.